1. The Math Calculations

MIN (Fastest Overflow - using Prescaler 1):

When there is no prescaling, the timer receives 1,000,000 ticks per second (1 tick = 1μs).

- Tmin=1×2561,000,000 Hz=0.000256 seconds

- MIN => 256 μs (microseconds)

MAX (Slowest Overflow - using Prescaler 1024):

When using the maximum prescaler of 1024, the timer receives only 1,000,000/1024=976.56 ticks per second (1 tick = 1.024ms).

- Tmax=1024×2561,000,000 Hz=262,1441,000,000=0.262144 seconds

- MAX => 262.144 ms (milliseconds)

2. The Extension Question

How can you extend the maximum real time measured in NORMAL MODE?

You can extend the maximum measurable time by implementing a Software Counter inside the Interrupt Service Routine (ISR). By declaring a global volatile variable and incrementing it every time the overflow interrupt triggers, you can count multiple hardware overflows to measure seconds, minutes, or even hours.

Example: If the hardware overflows every 262.144ms, counting 4 overflows in software gives you approximately 1 second of real time (4×262.144ms=1.048s).